Water Quality Monitoring System

Raspberry pi Mini Project Report

Using: Raspberry Pi, ADS1115 ADC, TDS & Turbidity Sensors, LED and email Alert & Visualization

| Name | Roll No. | Branch | Semester | Project Type | Academic Year |
| --- | --- | --- | --- | --- | --- |
| Anulipi Jana | 25-15-02 | Applied Physics | II Sem MTech | Mini Project | 2025–26 |

# 1.  Abstract

Access to clean water is fundamental to human health. This project presents a low-cost, real-time water quality monitoring system built around a Raspberry Pi single-board computer. The system acquires analog sensor data—Total Dissolved Solids (TDS) and turbidity—through an ADS1115 16-bit ADC over the I²C bus, computes quality metrics using calibrated polynomial equations, and displays live readings on a 16×2 LCD. An LED-based alert sub-system provides immediate visual feedback when measured parameters exceed safe thresholds. The integrated solution delivers actionable water quality information suitable for rural, domestic, and field deployment scenarios.

# 2.  Introduction

Embedded IoT-based systems, portable, affordable, and capable of delivering real time continuous data without specialist intervention.

This project targets two of the most critical water quality indicators:

- Total Dissolved Solids (TDS) – quantifies dissolved salts, minerals, and organic matter (measured in ppm).

- Turbidity – measures the optical clarity of water (measured in NTU). High turbidity signals suspended particles, sediment, or biological contamination.

By combining these two sensors with a Raspberry Pi and a real-time LCD display , visualization and LED alert system, the project provides a complete pipeline from raw analog sensing to human-interpretable output.

# 3.  System Architecture

## 3.1  Hardware Components

| Component | Module / Part | Role in System |
| --- | --- | --- |
| Microcontroller | Raspberry Pi | Central processing, I²C master, GPIO control |
| ADC | ADS1115 (16-bit) | Converts analog sensor voltages to digital values |
| TDS Sensor | Gravity TDS Probe | Measures dissolved solids in water (ppm) |
| Turbidity Sensor | SEN0189 / analog | Measures water clarity (NTU) |
| Display | 16×2 LCD (I²C PCF8574) | Shows real-time TDS, turbidity, and status |
| Alert System | RGB / Tri-colour LEDs | Green = safe, Yellow = caution, Red = unsafe |
| Power Supply | 5 V / 3 A USB-C | Powers RPi and peripherals |
| Connection | Breadboard & Jumper wires | Make connection between all devices. |

## 3.2  I²C Bus Configuration

Both the ADS1115 ADC and the LCD backpack (PCF8574) share the Raspberry Pi's I²C bus (GPIO 2 – SDA, GPIO 3 – SCL). The ADS1115 occupies address 0x48 (default) and the LCD occupies 0x27. The turbidity sensor is connected to ADS channel P0 and the TDS sensor to channel P1.

## 3.3 TDS sensor

It measures the water's electrical conductivity (EC) and converts it to a TDS value using a conversion factor.

Working Principle: When inorganic salts (like NaCl, CaCO₃) dissolve in water, they split into ions (positively charged cations & negatively charged anions). The sensor has two electrodes immersed in water. A voltage is applied, and the current flowing between them is measured. More ions = higher conductivity. Pure water has very low conductivity; saltwater has high conductivity .

## 3.4 Turbidity Sensor

Use optical phenomena to quantify suspended particles scatter light to measures water clarity.

Working Principle: Light source and photodetector positioned at 90° angle. Measures light scattered sideways. Receiver detects light reflected/back scattered by particles .Measures light absorbed/transmitted through the sample. more light absorbed = higher turbidity

## 3.5  LED Alert Logic

Three GPIO-controlled LEDs provide a colour-coded visual alert layer on top of the LCD display:

| LED Colour | Water Status |
| --- | --- |
| Green | Safe |
| BLUE | Polluted |

4.  Software Design

## 4.1  Libraries & Dependencies

The system software is written in Python 3 and relies on the following libraries:

| Library | Purpose |
| --- | --- |
| adafruit-circuitpython-ads1x15 | ADS1115 ADC driver (I²C) |
| RPLCD | 16×2 I²C LCD character display driver |
| RPi.GPIO | GPIO control for LED outputs |
| board / busio | Circuit Python hardware abstraction layer |
| time.sleep | Loop delay for 2-second sampling interval |
| matplotlib | Graph plot to show how the tds and turbidity change with water quality. |

## 4.2  Signal Processing & Calibration

Raw voltage readings from the ADS1115 are converted to physical units using experimentally derived polynomial equations:

TDS (ppm) = (133.42 × V³ – 255.86 × V² + 857.39 × V) × 0.5 Turbidity (NTU) = –1120.4 × V² + 5742.3 × V – 4352.9

where V is the measured voltage in volts.

## 5. Circuit Connection

| Rpi | ADS1115 | TDS (ppm) | Turbidity (NTU) | LED | LCD Status |
| --- | --- | --- | --- | --- | --- |
| Power 5V |  | VCC | VCC | Power | VCC |
| GND | GND | GND | GND | GND | GND |
| SCL (Pin 5) | SCL |  |  |  | SCL |
| SDL (Pin 3) | SDL |  |  |  | SDL |
| Power 3V | VDD |  |  |  |  |
|  | A0 |  | A0 |  |  |
|  | A1 | A1 |  |  |  |
| Pin 11 |  |  |  | GREEN |  |
| Pin 13 |  |  |  | BLUE |  |

## 6. Circuit Loop

The integrated main loop (combined from tds_check.py and turb_check.py) executes the following steps every 2 seconds:

- Read TDS voltage from ADS1115 channel P1 and turbidity voltage from channel P0.

- Compute TDS (ppm) and turbidity (NTU) using the calibration equations above.

- Classify water status: No Water / Clean / Dirty / Salty for TDS; Clear / Cloudy / Turbid for turbidity.

- Drive the appropriate LED (Green / Yellow / Red) via GPIO based on combined thresholds.

- Update the 16×2 LCD — Line 1: sensor values; Line 2: combined status string.

- Print a formatted debug summary to the serial console for logging and graphical representation.

# 7.  Results & Observations

# 8.  Challenges

8.1> Rpi is slower than other processor. Take multiple attempts just to connect properly.

- Patient

8.2> Compatibility of each device as they have difference version, port, IP addresses.

- version check before write code. If mismatch install properly.

8.3> As there are multiple components connected on a single chip difficult to find out issues on HW or software.
		- Divide into modules check each and every code individually then combine them.

8.4> Installation problem. Libraries like Pandas, LCD drive, adafruit-circuitpython-ads1x15 some time create path, version problems also take a lot of time.

8.5> Due to voltage or manufacturing or other reason Sensor’s output range became very small. 0.001 difference.

- Calibration

# 9.  Conclusion & Future Scope

The Water Quality Monitoring System successfully demonstrates real-time, multi-parameter water quality assessment using low-cost embedded hardware. The combination of TDS and turbidity measurements with an LED alert system provides an intuitive, actionable output suitable for non-expert users.

Planned enhancements for future iterations include:

- Addition of pH and dissolved-oxygen sensors for a more comprehensive water quality index.

- Wi-Fi/MQTT data uplink to an IoT dashboard (e.g., ThingSpeak or Node-RED) for remote monitoring and historical trending.

- Buzzer-based audible alerts for environments where visual LEDs may be missed.

- Machine-learning-based anomaly detection trained on historical sensor data to distinguish contamination events from sensor drift.

- Waterproof enclosure design with IP67 rating for outdoor or submerged deployment.